JMYReader.DLL Instructions for use

导出函数共7个（64位版本与32位版本相同）

There are 7 exported functions (the 64-bit version is the same as the 32-bit version)

the same as the 32-bit version)

函数说明(Function description)：

1，获取动态库版本(Get the dynamic library version)

int __stdcall JMY_GetVersion(unsigned char *verInfo);

verInfo需要足够长的空间，目前直接返回字符串 “JmyReaderDLL(01.24)”

(verInfo needs a long enough space, currently directly returns the string "JmyReaderDLL(01.24)")

2，获取可用端口列表(Get a list of available ports)

int __stdcall JMY_GetPortList(OUT int* numOfCom, OUT int* numOfUsbHid)；

如果要使用UsbHid接口类型的读卡器，必须先调用此函数，每次重新插拔都需要调用此函数，此函数内部枚举可用的串口列表，以及UsbHid列表；动态库，需要通过枚举UsbHid设备保存设备的路径，然后才能通过JMY_OpenPort函数打开UsbHid类型的读卡器

(If you want to use the card reader with UsbHid interface type, you must call this function first, and you need to call this function every time you replug it. This function internally enumerates the list of available serial ports and the list of UsbHid; dynamic library, you need to enumerate UsbHid device and saves the path of the device, and then the UsbHid type card reader can be opened through the JMY_OpenPort function)

3，获取端口属性，端口号和友好名称(Get port properties, port number and friendly name)

int __stdcall JMY_GetPortAttributes(int idx, int* portNum, char* friendName)；

该函数一般用于具有UI的程序，需要显示可用端口列表时使用，不需要显示则不需要使用该函数；

(This function is generally used for programs with UI, and it is used when the list of available ports needs to be displayed, and this function does not need to be used if it does not need to be displayed;)

idx参数为获取所需端口的索引号，该值如果是串口，则是从 0 到 numOfCom-1，如果是UsbHid则是 100 到 100+numOfUsbHid-1；

(The idx parameter is the index number of the required port. If it is a serial port, the value is from 0 to numOfCom-1, and if it is UsbHid, it is 100 to 100+numOfUsbHid-1;)

4， 获取错误代码的字符串(String to get the error code)

void __stdcall JMY_ErrorString(int err, char*errmsg)；

该函数也是用于具有UI程序的显示，可以不需要使用该函数；

(This function is also used for displaying with UI programs, so you don’t need to use this function;)

5，打开端口(open port)

int __stdcall JMY_OpenPort(int nPort, int nBaud)；

nPort为端口号，如果是串口则是实际的端口号，如果是UsbHid则，首先需要调用JMY_GetPortList函数，根据获得的numOfUsbHid的数量，第一个，端口号为 100，第二个端口号为101，依次类推；

(nPort is the port number. If it is a serial port, then it is the actual port number. If it is UsbHid, you first need to call the JMY_GetPortList function. According to the number of numOfUsbHid obtained, the first port number is 100, and the second port number is 101. And so on;)

nBaud，如果为串口则为下列值之一 { 9600, 19200, 38400, 57600, 115200 };

(nBaud, one of the following values if it is a serial port { 9600, 19200, 38400, 57600, 115200 };)

如果为UsbHid，把nBaud 设置为 MagicNumber 表示要求内部使用SetOutputReport/GetInputReport API，否则使用ReadFile/WriteFile Api，两者对读卡器的数据传输实现方式有不同的要求； MagicNumber = 0x20220529；

(If it is UsbHid, setting nBaud to MagicNumber means that the internal use of SetOutputReport/GetInputReport API is required, otherwise ReadFile/WriteFile Api is used, and the two have different requirements for the implementation of data transmission of the card reader; MagicNumber = 0x20220529;)

6，关闭端口(close port)

int __stdcall JMY_ClosePort(int nPort)；

关闭端口，nPort应为打开端口所使用的端口号；

(Close the port, nPort should be the port number used to open the port;)

7，执行指令(execute command)

int __stdcall JMY_ExecuteCmd(int nPort, unsigned short Addr, unsigned char Cmd, unsigned char *Data, int* DataLen, int Timeout)；

nPort为打开的端口号

(nPort is the open port number)

Addr  将该值设置为大于 0xFF的值，例如0x100，函数内部使用JCP04协议封装数据包，该值的范围如果为 0x00 ～ 0xFF，则函数内部使用JCP05协议封装数据包。

(Addr sets the value to a value greater than 0xFF, such as 0x100, and the function internally uses the JCP04 protocol to encapsulate the data packet. If the value ranges from 0x00 to 0xFF, the function internally uses the JCP05 protocol to encapsulate the data packet.)

Cmd为命令码，根据金木雨的读卡器手册设置

(Cmd is the command code, set according to Jinmuyu's card reader manual)

Data为数据指针， 是输入数据，也是输出数据的Buffer，所以，需要足够的空间，一般为512字节

(Data is the data pointer, which is the input data and the buffer of the output data, so enough space is needed, generally 512 bytes)

DataLen表示Data的长度，读卡器返回的数据长度也将赋值给该变量；

(DataLen indicates the length of Data, and the length of data returned by the card reader will also be assigned to this variable ;)

Timeout为需要等待读卡器数据的最长等待时长，毫秒。

(Timeout is the longest waiting time for data from the card reader, in milliseconds.)

关于返回值，错误值ERR_CRCERR和ERR_CMDFAIL表示IO正确，但是数据检查错误，CRCERR表示校验字节不符合计算要求（除CRC字节外的所有字节的Xor值），CMDFAIL表示读卡器执行命令出错！  OPENREADER表示打开设备句柄发生错误。 DATAOUT表示写数据到设备句柄发生超时以外的其他错误， DATAIN表示从设备读数据时，发生了超时以外的其他错误。TIMEOUT表示读写数据超时！

(Regarding the return value, the error values ERR_CRCERR and ERR_CMDFAIL indicate that the IO is correct, but the data check is wrong, CRCERR indicates that the checksum byte does not meet the calculation requirements (Xor value of all bytes except the CRC byte), and CMDFAIL indicates that the card reader executes the command error! OPENREADER indicates that an error occurred opening the device handle. DATAOUT indicates an error other than a timeout occurred when writing data to the device handle, and DATAIN indicates an error other than a timeout occurred when reading data from the device. TIMEOUT means reading and writing data timed out!)

建议发送读卡器有可能很长时间执行的命令之前，先执行一个执行较短时长的指令（0015000001 读一字节Flash内容），用来明确通信是正常，以避免不必要的等待。

(It is recommended to execute a command with a shorter execution time (0015000001 to read one byte of Flash content) before sending a command that the card reader may execute for a long time, to make sure that the communication is normal and to avoid unnecessary waiting.)

使用JMYReader.dll 不需要头文件，不需要lib文件，使用以下代码（需VS2013以上，支持C++11标准）即可

(Using JMYReader.dll does not require header files or lib files, just use the following code (requires VS2013 or above, supports C++11 standard))

#include <Windows.h>

#include <functional>

#include <string>

//JMYReader Dll Declare

/*

int __stdcall *JMY_GetVersion(OUT unsigned char * verInfo);

int __stdcall *JMY_GetPortList(OUT int* numOfCom, OUT int* numOfUsbHid);

int __stdcall *JMY_GetPortAttributes(int idx, int* portNum, char* friendName);

int __stdcall *JMY_OpenPort(int nPort, int nBaud);

int __stdcall *JMY_ClosePort(int nPort);

int __stdcall *JMY_ExecuteCmd(int nport, unsigned short Addr, unsigned char Cmd, unsigned char* Data, int* DataLen, int timeout);

void __stdcall *JMY_ErrorString(int err, char*errmsg);*/

const int ERR_SUCCEED = 0;

const int ERR_PARAM = -1;

const int ERR_WTIMEOUT = -2;

const int ERR_RTIMEOUT = -3;

const int ERR_OPENREADER = -4;

const int ERR_DATAOUT = -5;

const int ERR_DATAIN = -6;

const int ERR_CRCERR = -7;

const int ERR_CMDFAIL = -8;

static HMODULE hJMYReader = LoadLibrary(L"JMYReader.dll");

template <typename T>

std::function<T> GetFunc(const std::string &funcName)

{

FARPROC funAddress = GetProcAddress(hJMYReader, funcName.c_str());

if (funAddress == NULL)

{

CString szMsg;

CString szfunc = CString(funcName.c_str());

szMsg.Format(L"Load JMYReader.DLL failed! func<%s> cann't be load!", szfunc);

AfxMessageBox(szMsg);

exit(-1);

}

return std::function<T>((T*)(funAddress));

}

#define F(funcName,typefunc) static auto funcName = GetFunc<typefunc>(#funcName)

F(JMY_GetVersion, int __stdcall (unsigned char* verInfo));

F(JMY_GetPortList, int __stdcall (int* numOfCom, int* numOfUsbHid));

F(JMY_GetPortAttributes, int __stdcall (int idx, int* portNum, char* friendName));

F(JMY_OpenPort, int __stdcall (int nport, int nBaud));

F(JMY_ClosePort, int __stdcall (int nport));

F(JMY_ExecuteCmd, int __stdcall (int nport, USHORT Addr, BYTE Cmd, BYTE* Data, int* DataLen, int timeout));

F(JMY_ErrorString, void __stdcall (int err, char* errMsg));

就可以在程序中调用相应的函数

(You can call the corresponding function in the program)